Assignment No 1

Cs610 (Computer Networks)

Name: Abdullah

Student ID: BC230413824

Question # 1 Solution:

- What could be causing the high collision rate in this setup?

- The network is using a hub, not a switch, In a hub-based 10Base-T Ethernet network, all devise share the same collision domain, meaning only one device can successfully transmit at a time.

- 10Base-T uses CSMA/CD (Carrier Sense Multiple Access with Collision Detection), but heavy traffic makes collisions more frequent.

- More users and heavier traffic (e.g. transferring large files) increase the change of collations, leading to retransmission and slowing down the network.

- Why might frames be getting dropped due to unknown types?

- Frames can be dropped if they contain protocol types that the receiving devices or network equipment (like the sniffer or old network cards) do not recognize.

- Possible reason:

  - Misconfigured devices using non-standard or proprietary protocols.

  - Unsupported protocols being used on the network that the monitoring tool or some network devices cannot interpret.

- How can a network analyzer help resolve these issues?

A network analyzer (sniffer) can help resolve these issues by:

Quantifying collision rates: Providing precise measurements of collision       frequency and identifying peak collision periods

Protocol analysis: Identifying the specific unknown protocols causing frame drops and their source devices

Traffic pattern identification: Revealing which devices or applications are generating excessive traffic leading to congestion

Collision detection: Identifying malformed packets, CRC errors, or other transmission problems

Baseline establishment: Creating a performance baseline to measure improvements after implementing solutions

Based on this data, the technician can recommend solutions, such as:

- Replacing the hub with a switch (to create separate collision domains).

- Updating device configuration to ensure standard protocols are used.

- Replacing faulty network cables or NICs.

Question # 2 Solution:

- Which Ethernet technology would you recommend for this setup and why?

I recommend using 10Base-T Ehernet with hubs for this office setup.

Reasons:

- Cost-effectiveness: 10Base-T uses twisted pair cabling which is less expensive than coaxial cable used in Thick or Thin Ethernet

- Modularity: Star topology allows for easy addition or removal of computers without disrupting the entire network

- Fault isolation: Cable failures in 10Base-T only affect the individual connection between a computer and the hub, not the entire network

- Maintenance simplicity: Problems can be diagnosed and fixed quickly by checking individual connections.

- What kind of NIC would be most suitable in this case?

The most suitable NIC would be an Ethernet NIC with an FJ-45 connector that support 10 Mbps (10Base-T)

- These NICs are inexpensive and widely available.

- The allow direct connection to the twisted-pair cabling and hubs used in a 10Base-T network.

- What are the limitations of the technologies you did not choose?

| Technology | Limitations |
| --- | --- |
| Thick Ethernet (10Base5) | Very expensive and bulky cabling. Difficult to install and maintain. Requires vampire tabs for connections. A single cable fault can take down the entire network segment. |
| Think Ethernet (10Base2) | Still uses coaxial cable, which is harder to manage, than twisted pair. Bus topology means a cable break affects all computers on the segment. Limited flexibility for network expansion. Outdated technology, not suitable for modern office environments. |